Placement algorithms in FPGAs

What is it?

- Placement algorithm defines that which logic block of a netlist maps on which CLB of FPGA.

Optimization goal

- To place connected logic blocks of the netlist near so that minimum FPGA routing resources are used.

- To place the logic blocks in such a way that circuit speed is increased.

- To place the logic blocks in such a way that wire density across FPGA is balanced.

Verilog -> Netlist consists of 9 CLBs, FPGA consists of 9 CLBs.  9!

Netlist 64 CLB, FPGA 64 CLBs…. 64! -> 1.2 x 10^89 -> 1.2 x 10^83 seconds/3600*24*365

= 1.2 x 10^75 years are required

1 million placement combinations are explored in 1 second

Classes of Placement algorithms

- Three major classes of placement algorithms used in industry

  - Min-Cut (Partitioning based)

  - Analytical + local iterative improvement

  - Heuristic algorithms: Simulated annealing based placement algorithms.

Physical phenomenon

- Gradually cool molten metal to produce high-quality metal objects.

Cost function = Wire length based cost function = Sum of the half perimeters of the bounding boxes of all the nets in a netlist

Psuedo-code of a generic simulated annealing-based placer.

Initial Temperature measurement

- Let N is the total number of blocks and IO in a netlist.

- Create a random placement of all the netlist instances.

- Perform N random moves

- Compute standard deviation of the cost of these Nblocks different configurations.

- Initial temperature is set to be 20 times standard deviation.

Number of Move operations in a  temperature step:

Default Inner Num = 10, Reducing InnerNum by a factor of 10 reduces the placement quality by 10%, and reduces execution time by 10x.

Initial Placement

- Random initial placement.

Cost Function

- Wirelenght driven placement has the following cost function.

  - Sum of half perimeters of the bounding boxes of all the nets in the netlist.

Exponent function

- e0 = 1…,

- e0.1 = 1.105…,

- e0.2 = 1.22…,

- e-1 = 0.378…,

- e-0.0001 = 0.9999…

- e1 = 2.7182

- e12 = 162,754.791

- If( r < e-dC/T ) accept it, else reject it. means that

  - When dC is negative (i.e. cost has improved), exponent will be very high. Thus move will be accepted.

  - When dC is positive (i.e. cost has increased), and temperature is high, the move can be accepted.

- Escape local minimum solution.

Exit criteria or Termination condition of the outer loop is given below

Use ɛ = 0.005

UpdateTemperature()

- Here α = acceptance percentage (total percentage of moves accepted)

- The rate at which temperature is decreased.

UpdateRLimit()

- It is desired to keep the total number of accepted moves to be 44 percent.

- α is used to control a range limiter.

Bounding Box Computation

- For all the nets in the netlist

  - Compute the bounding box of the net

    - For all the pins of a net, find the maximum and minimum position of x and y coordinate.

    - Compute the BBX value.

Bounding Box Computation – optimization

- If two blocks are swapped, the BBX cost of the nets attached with these blocks need to be updated.

Incremental Net Bounding Box Updates

- The function that computes change in cost must be optimized.

- If two blocks are swapped, the BBX cost of the nets attached with these blocks need to be updated.

- For each net,

  - Store the following four coordinates of the BBX

    - Xmin, Xmax, Ymin, Ymax

  - Store the number of net terminals lying on each sides

    - Nxmin, Nxmax, Nymin, Nymax.

  - The figure below shows this case

- When a net terminal moved from (xold, yold) to (xnew, ynew), we can determine the new bounding box cost function only by looking at the terminal that moved, rather than by looking at all the terminals of the net.

- Following pseudo-code is used

- The bruteforceBoundingBoxRecompute function is only called when

  - The terminal moved is the only terminal on that side of the bounding box.

  - And it is moved inwards, towards the bounding box center.

- Repeat the above cases for

  - Xmax

  - Ymin

  - Ymax